Word Recognition System

Speech-Command Classification with MFCC + GMM

Advanced DSP — Project Report

Figure 1. End-to-end system architecture

.

1. Overview

This project presents a word-recognition system that classifies short audio inputs into three categories: "Light On", "Light Off", and "Other". The recognised command drives a simulated light through a real-time graphical interface, demonstrating a complete voice-controlled pipeline from microphone to action.

The signal-processing pipeline consists of three main stages:

- Pre-Processing — Pre-Emphasis to boost high frequencies, Voice Activity Detection (VAD) to isolate speech, and Hamming windowing to divide the signal into short frames.

- Feature Extraction — Mel-Frequency Cepstral Coefficients (MFCCs) together with their first and second derivatives, giving 39 features per frame.

- Model Training — Gaussian Mixture Models (GMMs) that classify new audio based on maximum log-likelihood.

Dataset

The system was trained on 308 "Light On", 306 "Light Off" and 602 "Other" samples. Testing used a balanced set of 60 samples per category.

2. Signal Processing Pipeline

2.1 Configuration & Loading

Audio files are read per category with audioread, preserving each file’s native sample rate for downstream processing.

clc; clear; close all; % Define paths basePath = fullfile('Data', 'Train'); pathOn = fullfile(basePath, 'LightOn'); pathOff = fullfile(basePath, 'LightOff'); % Get first wav file from each folder filesOn = dir(fullfile(pathOn, '*.wav')); filesOff = dir(fullfile(pathOff, '*.wav')); % Read audio files [audioOn, fsOn] = audioread(fullfile(pathOn, filesOn(1).name)); [audioOff, fsOff] = audioread(fullfile(pathOff, filesOff(1).name));

2.2 Pre-Emphasis

% Apply pre-emphasis alpha = 0.97; preEmphOn = filter([1, -alpha], 1, audioOn); preEmphOff = filter([1, -alpha], 1, audioOff); % Compute pre-emphasized FFT fftOn_pre = abs(fft(preEmphOn)); fftOff_pre = abs(fft(preEmphOff));

Pre-emphasis applies a first-order high-pass filter (alpha = 0.97) to the raw signal. This amplifies high frequencies that are naturally attenuated in human speech, producing a more balanced spectrum and improving the quality of the extracted features.

2.3 Voice Activity Detection (VAD)

% Define window length for energy envelope winLen = round(0.02 * fs); % Calculate signal envelope using moving average env = movmean(abs(preEmphSignal), winLen); % Define threshold and build the VAD mask thresh = 0.1 * max(env); vadMask = env > thresh; % Keep only active speech vadSignal = preEmphSignal(vadMask);

VAD isolates speech segments by removing silence and ambient noise. A moving average of the signal magnitude forms an energy envelope; frames whose envelope exceeds 10% of the peak are kept as active speech. This ensures the feature-extraction stage processes only relevant acoustic data and prevents the model from learning noise characteristics.

2.4 Framing & Windowing

Because speech is non-stationary, it is analysed in short (~25 ms) overlapping frames. Abrupt frame boundaries introduce spectral leakage, so a Hamming window tapers each frame toward its edges, yielding a more reliable frequency representation before the FFT.

% Define frame parameters frameDuration = 0.025; % 25 ms frames winLen = round(frameDuration * fs); % Create Hamming window and apply it to a frame hamWin = hamming(winLen); frameWin = frameOrig .* hamWin;

3. Feature Extraction (MFCC)

To represent voice commands mathematically, the system extracts Mel-Frequency Cepstral Coefficients. Each windowed frame is transformed to the frequency domain, passed through a Mel-scale filterbank, log-scaled, and reduced with a DCT. The first 13 coefficients describe the vocal-tract shape; appending Delta and Delta-Delta coefficients expands the representation to 39 features per frame, capturing how the spectrum evolves over time.

% Frame parameters winLenMfcc = round(0.025 * fs); % 25 ms window overlapMfcc = round(0.015 * fs); % 15 ms overlap window = hamming(winLenMfcc); % Extract MFCC + first and second derivatives -> 39 features/frame [coeffs, delta, deltaDelta, ~] = mfcc(vadSignal, fs, ... 'Window', window, 'OverlapLength', overlapMfcc); features = [coeffs, delta, deltaDelta];

4. Model Training (GMM)

Three separate Gaussian Mixture Models — one per class, each with 16 mixture components — model the 39-dimensional feature distribution of "Light On", "Light Off" and "Other". A small regularisation value keeps the covariance estimates numerically stable. The trained models are saved to a .mat file for real-time classification and deployment.

numMixtures = 16; options = statset('MaxIter', 200, 'Display', 'final'); % Train one GMM per class on the pooled 39-D feature frames gmmOn = fitgmdist(allFeatures{1}, numMixtures, ... 'Options', options, 'RegularizationValue', 0.01); gmmOff = fitgmdist(allFeatures{2}, numMixtures, ... 'Options', options, 'RegularizationValue', 0.01); gmmOther = fitgmdist(allFeatures{3}, numMixtures, ... 'Options', options, 'RegularizationValue', 0.01); % Save models for deployment save(fullfile('models','trained_gmm_models.mat'), ... 'gmmOn', 'gmmOff', 'gmmOther');

5. Testing & Evaluation

During testing, each unseen clip passes through the same pipeline. Its feature frames are scored under all three GMMs using the total log-likelihood, and the clip is assigned to the highest-scoring model. A small constant (1e-10) is added inside the logarithm to avoid taking log(0).

% Log-likelihood of the clip under each model scoreOn = sum(log(pdf(gmmOn, features) + 1e-10)); scoreOff = sum(log(pdf(gmmOff, features) + 1e-10)); scoreOther = sum(log(pdf(gmmOther, features) + 1e-10)); % Predicted class = model with the highest score [~, predIdx] = max([scoreOn, scoreOff, scoreOther]);

The system achieves an overall accuracy of 88.27% on the balanced test set. The confusion matrix shows strong recognition of "Light On" and "Other", while "Light Off" is the most challenging class — most often confused with "Light On", which is acoustically similar.

Figure 2. Confusion matrix on the test set (39 features).

| Class | Recall (Accuracy) | Note |
| --- | --- | --- |
| Light On | 96.7% | Strongest class |
| Light Off | 75.0% | Most challenging |
| Other | 93.2% | Strong |
| Overall | 88.27% | Balanced test set (60/class) |

6. Real-Time GUI

A MATLAB App Designer interface deploys the trained models for live use. Pressing START records two seconds of microphone audio, plots its time- and frequency-domain views, and runs the full pre-processing and MFCC pipeline. The three GMM scores are converted into a softmax confidence percentage; the recognised command then drives a virtual light (LED lamp) and is written to a system log.

Figure 4. Real-time GUI — recognition output.

7. Conclusion

The project delivers a complete, working voice-command recognition system: a principled DSP front end (pre-emphasis, VAD, windowing), a compact 39-dimensional MFCC feature set, and per-class Gaussian Mixture Models combined with a maximum-likelihood decision rule. At 88.27% overall accuracy with a responsive real-time interface, it demonstrates that classical DSP and statistical modelling remain an effective, transparent approach to small-vocabulary speech recognition. The clearest avenue for improvement is separating the two similar commands, for example with more discriminative features or additional training data for "Light Off".